ELTE IK – GPU Programming 2025/2026/2

Random Number Generation

Leszkovszky Alexandra, Kaselyák Péter, Márton Gergő

1.	Project Overview	2

1.1.	Hardware Description	2

2.	Design Considerations	2

2.1.	Local Memory Staging	2

2.2.	Workgroup Size Selection	2

2.3.	Per-Thread Seed Independence	3

2.4.	Mersenne Twister Design	3

2.5.	Monte Carlo Kernel Design	3

3.	Performance Analysis	3

3.1.	RNG Throughput – N = 262 144	3

3.2.	RNG Throughput – N = 524 288	4

3.3.	CPU vs. GPU Comparison	4

4.	Statistical Testing – Chi-Square Uniformity	4

5.	Monte Carlo Simulations	5

5.1.	Pi Estimation	5

5.2.	S&P 500 Stock Option Probability	5

6.	Code Architecture	6

- Project Overview

This project implements three GPU-accelerated random number generators (RNGs) in OpenCL. There are the Linear Congruential Generator (LCG), XORShift, and Mersenne Twister (MT19937) generators. Their outputs are used to drive two Monte Carlo simulations: a Pi estimation and an S&P 500 stock option probability model. All generators run in parallel across 1024 work-items, each producing 256 values, for a total of 262,144 random numbers per generator per run.

  - Hardware Description

All the stated values in the document are relevant for the executing hardware only. This was an ASUS TUF Gaming A15 laptop with an AMD Ryzen 5 4600H CPU and an NVIDIA GeForce RTX 3050 GPU.

- Design Considerations

  - Local Memory Staging

The core optimization applied to all three RNG kernels is local memory staging of output. Without this, each work-item writes its generated value directly to a unique location in global memory, producing N scattered write transactions per iteration. That is one per work-item.

The optimized approach uses a local staging buffer of LOCAL_SIZE (64) slots. Each work-item writes to localBuf[localId] instead of directly to global memory. After a CLK_LOCAL_MEM_FENCE barrier ensures all lanes have written, the first thread/work-item performs a single sequential flush of the entire workgroup slice to the correct position in global memory.

This reduces global memory write transactions by a factor of LOCAL_SIZE. The writes become coalesced, meaning consecutive lanes write to consecutive addresses. This maps efficiently onto the GPU memory controller's burst transfer model.

  - Workgroup Size Selection

The value 64 was chosen for LOCAL_SIZE because it is a multiple of NVIDIA’s Warp (32) and AMD’s Wavefront (64) sizes. These are equal terms for how the hardware operates. When a workgroup is dispatched to a Streaming Multiprocessor, its work-items are split up into groups of Warp/Wavefront size, and these groups are executed in SIMD (Single Instruction Multiple Data) format. This way we can avoid partial Warp/Wavefront waste. Larger sized workgroups would reduce the number of resident workgroups, wasting hardware capacity, while smaller sizes would waste barrier synchronization overhead.

  - Per-Thread Seed Independence

Each work-item uses a seed constructed from a base seed, Knuth’s multiplicative constant (2654435761) and its global ID.

// LCG
unsigned long state = seed + (unsigned long)globalId * 2654435761UL;

// XORShift  
unsigned int state = seed ^ (globalId * 2654435761U + 1U);

// MT
mt[0] = seed ^ (globalId * 2654435761U);

  - Mersenne Twister Design

The Mersenne Twister generator uses an array of 624 integers as its internal state. This means that per thread/work-item, MT uses 624 × 32 bits (or 624 × 4 bytes) of space for only storing the state. Using local memory, this implementation would need 624 × 4 = 2 496 bytes per work-item. With 64 work-items per workgroup that is 2 496 × 64 = 159 744 bytes. That is roughly 159KB. On the RTX 3050 graphics card each Streaming Multiprocessor has 48KB of local memory available, which is obviously much less than the needed 159KB. This disqualifies the usage of local memory, leaving the slower global memory storage the only option.

  - Monte Carlo Kernel Design

The Monte Carlo kernels consume any pre-generated flat buffer using the runMonteCarlo() function. Each Monte Carlo kernel utilizes the previously mentioned local memory buffering, minimizing the time needed for global memory writes.

- Performance Analysis

All measurements were taken on the previously mentioned hardware. GPU timing uses CL_QUEUE_PROFILING_ENABLE with cl_ulong nanosecond timestamps ensuring platform-independent, consistent 64-bit precision. clFinish() is called after each blocking read to guarantee timestamp commitment before querying the event.

  - RNG Throughput – N = 262 144

| Generator | GPU Time (ms) | Throughput (M/ms) |
| --- | --- | --- |
| LCG | 0.214 | 1224.88 |
| XORShift | 0.224 | 1168.95 |
| MT (19937) | 0.386 | 679.05 |

  - RNG Throughput – N = 524 288

| Generator | GPU Time (ms) | Throughput (M/ms) |
| --- | --- | --- |
| LCG | 0.420 | 1248.78 |
| XORShift | 0.423 | 1239.71 |
| MT (19937) | 0.651 | 805.03 |

LCG and XORShift achieve comparable throughputs because both require minimal per-step arithmetic and fit their entire state in a single register. MT’s throughput is approximately 55% of the other two’s mostly because of the 624-word state array in global memory and periodic twist recurrence, which requires 624 dependent read-modify-write operations before the next batch of outputs is available.

  - CPU vs. GPU Comparison

| Generator | Time (ms) | Throughput (M/ms) |
| --- | --- | --- |
| LCG | 0.420 | 1248.78 |
| XORShift | 0.423 | 1239.71 |
| MT (19937) | 0.651 | 805.03 |
| std::mt19937 (CPU) | 3.4562 | 75.8475 |

- Statistical Testing – Chi-Square Uniformity

The Chi-Square goodness-of-fit test measures whether the generated distribution deviates significantly from uniform. Bins are determined by Sturges' rule: K = 1 + floor(log2(N)). Bins with fewer than 600 observations are dropped before computing the statistic. The critical value at the 5% significance level for df = 18 is 28.87.

These statistics are for N = 262 144.

| Generator | Bins (K) | Expected Frequency | Chi-Sq Stat. | df | Pass |
| --- | --- | --- | --- | --- | --- |
| LCG | 19 | 13 797.1 | 14.288 | 18 | Yes |
| XORShift | 19 | 13 797.1 | 16.412 | 18 | Yes |
| MT (19937) | 19 | 13 797.1 | 21.281 | 18 | Yes |

All three GPU generators pass the Chi-Square test comfortably (Chi-Sq. Stat. < 28.87). LCG achieves the lowest statistic (14.288), indicating the most uniform distribution across bins at this sample size.

- Monte Carlo Simulations

  - Pi Estimation

The simulation determines how well does a set of random points in 2D space estimate Pi. At first, we normalize the points to [0, 1) and test whether x2+ y2 ≤1. The fraction of points inside the quarter-circle approximates to π4. In case of N = 262 144, our generators produced 131 072 pairs of numbers ((i, i+1)). The expected error from pure sampling variance is approximately 1N ≈0.0028.

| Generator | Estimated Pi | Abs. Error | MC Time (ms) |
| --- | --- | --- | --- |
| LCG | 3.13882 | 0.00277 | 0.044 |
| XORShift | 3.14490 | 0.00330 | 0.035 |
| MT (19937) | 3.13147 | 0.01012 | 0.034 |

All three generators produce estimates within the expected sampling error range. The Monte Carlo kernel itself runs in under 0.05 ms because it only reads from the pre-generated buffers provided by the generators.

  - S&P 500 Stock Option Probability

The stock kernel models an S&P 500 index fund using Geometric Brownian Motion. Two raw unsigned integers per path are converted to uniform floats, then transformed via Box-Muller to a standard normal variate Z. The terminal price is:

S_T = S0 * exp((mu - 0.5 * sigma^2) * T + sigma * sqrt(T) * Z)

Parameters: S0=5500; Strike=6200; mu=0.10; sigma=0.18; T=1.0 year

The analytical Black-Scholes probability PST>Strike=Phi(d2) where d2=lnS0K+mu-0.5* σ2*T  (σ* T)= -0.20028, giving Phi-0.20028=42.0593%.

| Generator | Strike Prob. | Abs. Err. vs. B-S. | MC Time (ms) |
| --- | --- | --- | --- |
| LCG | 42.0509 | 0.00836 | 0.098 |
| XORShift | 42.0128 | 0.04651 | 0.084 |
| MT (19937) | 41.9868 | 0.07245 | 0.079 |

LCG achieves the smallest absolute error (0.0084%) against the theoretical Black-Scholes value. All three generators produce results within 0.1% of the analytical solution, demonstrating that even the simplest GPU-friendly generator (LCG) provides adequate precision for financial simulations at this sample size.

- Code Architecture

| File | Responsibility |
| --- | --- |
| OpenCLUtils.hpp | checkError(), buildProgramWithLog(), CLKernel(), buildKernel(), profilingMs() |
| RngKernels.hpp | LCG, XORShift, MT19937 kernel source strings |
| MonteCarloKernels.hpp | Pi estimation and stock option kernel source strings |
| MonteCarloUtils.hpp | runMonteCarlo(): generic mechanical execution + readback |
| ChiSquareTest.hpp | runChiSquareTest(): Sturges binning + chi-square statistic |
| HistogramExport.hpp | exportHistogramCSV(): writes bin/count CSV for plotting |
| CpuBaseline.hpp | runCPUBaseline(): std::mt19937 timing + chi-square |
| main.cpp | Constants, OpenCL context, kernel launches, result interpretation |
| plot_histograms.py | matplotlib 2×2 histogram visualization from CSV files |